AWS Assignment Set 5 — Question 2

Linux Web Server on Amazon EC2 with Security Group

# 1. Objective

Deploy a Linux web server on Amazon EC2, secure it with an AWS Security Group, install Apache HTTP Server, host a custom webpage, and verify the deployment.

# 2. Environment

EC2 instance: Assignment5-Q2-WebServer

Instance type: t3.micro

Operating system: Ubuntu Server

Availability Zone: ap-south-1b

Web server: Apache2

Public IPv4 used for testing: 13.233.41.97

# 3. Security Group

| Protocol | Port | Source | Purpose |
| --- | --- | --- | --- |
| TCP | 22 | 0.0.0.0/0 | SSH remote administration |
| TCP | 80 | 0.0.0.0/0 | HTTP web traffic |
| TCP | 443 | 0.0.0.0/0 | HTTPS web traffic |

Security note: the captured configuration allows SSH from 0.0.0.0/0. For stronger security, SSH should be restricted to the administrator's trusted IP address or another controlled access mechanism.

# 4. Apache Deployment

Commands used:

sudo apt update

sudo apt upgrade -y

sudo apt install apache2 -y

sudo systemctl status apache2

Verification results:

sudo systemctl is-active apache2 → active

sudo systemctl is-enabled apache2 → enabled

curl -I http://localhost → HTTP/1.1 200 OK

sudo ss -tulnp | grep ':80' → Apache listening on TCP port 80

# 5. Custom Web Page

The custom webpage was deployed as /var/www/html/index.html and was successfully accessed through the EC2 public IPv4 address.

Displayed content:

Linux Web Server Successfully Deployed
AWS EC2 + Apache
Assignment Set 5 - Question 2

# 6. Architecture

Internet → Security Group (22/80/443) → EC2 Ubuntu → Apache2 → /var/www/html/index.html → Custom Web Page

# 7. Evidence

## OP-1.PNG

## OP-2.PNG

## OP-3.PNG

## OP-4.PNG

## OP-5.PNG

## OP-6.PNG

# 8. Conclusion

The Linux web server was successfully deployed on Amazon EC2 using Ubuntu and Apache. The Security Group was configured with SSH, HTTP, and HTTPS inbound rules. Apache was verified as active and enabled, HTTP port 80 was verified as listening, and the custom webpage was successfully served through the EC2 public IPv4 address.